# Plik mqtt_demo.py import _thread import esp32 import machine import network import time import wifi_config import sys from umqtt.robust import MQTTClient # 1 client_id = machine.unique_id().hex().upper() # 2 print(f"Client ID: {client_id}") def wifi_connect(): # 3 station = network.WLAN(network.STA_IF) station.active(True) if not station.isconnected(): print("Łączenie z siecią", end="") station.connect(wifi_config.ssid, wifi_config.password) while not station.isconnected(): print(".", end="") time.sleep_ms(250) print() print(f"Adres IP: {station.ifconfig()[0]}") def mqtt_publish(topic, payload): # 4 client.publish( f"0000000_Elektronika_Praktyczna/{topic}", payload # 5 ) def mqtt_callback(topic, payload): # 6 print(f"{topic} > {payload}") def mqtt_listener_task(delay_ms): # 7 while True: # 8 time.sleep_ms(delay_ms) # 9 try: client.check_msg() # 10 except Exception as e: sys.print_exception(e) def temperature_task(delay_ms): # 11 while True: client.publish( f"0000000_Elektronika_Praktyczna/sensor_{client_id}", # 12 str(esp32.mcu_temperature()) ) time.sleep_ms(delay_ms) wifi_connect() # 13 client = MQTTClient(client_id, "test.mosquitto.org") # 14 client.set_callback(mqtt_callback) # 15 client.connect() # 16 # client.subscribe("0000000_Elektronika_Praktyczna") # 17 client.subscribe("0000000_Elektronika_Praktyczna/test") # client.subscribe("0000000_Elektronika_Praktyczna/#") print("Oczekiwanie na wiadomości z MQTT") _thread.start_new_thread(mqtt_listener_task, [1000]) # 18 _thread.start_new_thread(temperature_task, [5000]) # 19

Listing 1. Kod pliku mqtt_demo.py