void PADC1_Initialize(void)

{

// ADSLP disabled; ADSIDL disabled; PWRLVL Low-Power mode; PUMPEN disabled; FORM Integer, Raw Data; ADCAL disabled; ADON enabled;

ADCON1 = (0x8000 | 0x01) & 0x7FFF; //Full power mode, Enable ADC later

ADCON2 = (0x4000 | 0x0300);      // NVCFG0 AVSS; PVCFG VREF+; REFPUMP disabled; BUFORG disabled;

ADCON2bits.BUFORG = 1;  // Result buffer organized as indexed mode

BUFCON1 = 0x00;   // BUFOE disabled; BUFSTBY Normal; BUFEN disabled; BUFSIDL disabled; BUFSLP disabled; BUFREF 1.2 V;

ADCON3 = 0x8004;      // SLEN3 disabled; SLEN2 disabled; SLEN1 disabled; ADRC RC clock; SLEN0 disabled; ADCS 8;

//Set Sample lists

// MULCHEN One at a time; CTMEN disabled; CM Matching is disabled; SLINT No interrupt; WM All conversion results saved; SAMC 29 tad; ASEN enabled;

ADL0CONH = 0x801D;

ADL0CONHbits.SLINT = 0x01; //interrupt after autoscan completion

// THSRC Buffer register; SLTSRC Manual Trigger:Single Trigger; SLEN enabled; SLENCLR disabled; SLSIZE 2; SAMP disabled;

ADL0CONL = (0x8001 & 0x7FFF) | 0x4000;  // open manual switch and Enable sample list later

//Set table registers

ADTBL0 =  0x2;   // UCTMU disabled; ADCH AN2; DIFF disabled;

ADTBL1 =  0x3;  // UCTMU disabled; ADCH AN3; DIFF disabled;

// Set table pointer registers

ADL0PTR =0;

ADL1PTR =0;

ADL2PTR =0;

ADL3PTR =0;

ACCONL = 0x00;      // TBLSEL ADTBL0; COUNT 0;

ACCONH = 0x00;      // ACIE disabled; ACEN disabled

ADCON1bits.ADON = 1;  //Enable ADC

while(!(PADC1_IsReadyForConversion()));      //Poll the ADREADY bit

ADCON1bits.ADCAL = 1; // Start calibration

while(!(PADC1_IsReadyForConversion())); //Poll the ADREADY bit

ADTMRPR = 0x00;  //Set ADC timer register

//Enable sample list

ADL0CONLbits.SLEN = 1; // Enable Sample list 1

ADL0CONLbits.SAMP = 0; // Close sample switch

}

Listing 1. Konfiguracja PADC

void PADC1_SampleList0ManualConversionStart(void)

{

ADL0CONLbits.SAMP = 1;

ADL0CONLbits.SAMP = 0;

}

Listing 2. Wyzwolenie konwersji z zerowej listy próbek

void  GetPDAC(uint16_t *buffer)

{

uint32_t buf_oversampling[2];

uint8_t i;

buf_oversampling[0] = 0;

buf_oversampling[1] = 0;

for (i= 0; i< 32; i++)

{

while(!(PADC1_IsReadyForConversion()));

PADC1_SampleList0ManualConversionStart();

while(!PADC1_IsBusyInConversion());

PADC1_SampleList0ConversionResultBufferGet(buffer, 0, 2);

buf_oversampling[0] = buf_oversampling[0]+ buffer[0];

buf_oversampling[1] = buf_oversampling[1]+ buffer[1];

}

buffer[0] = buf_oversampling[0] /32;

buffer[1] = buf_oversampling[1] /32;

}

Listing 3. Odczyt wyniku konwersji kanałów AN2 i AN3

bool PADC1_SampleList0ConversionResultBufferGet (uint16_t *buffer,  uint8_t tableRegIndex, u int8_t slSize)

{

uint8_t index;

bool dataValid = false;

if (ADSTATLbits.SL0IF !=  0)

{

For (index=0; index < slSize; index++)

{

buffer[index] = *((uint16_t *) & ADRES0 + tableRegIndex);

tableRegIndex++;

}

ADL0STATbits.ADLIF = 0;

dataValid = true;

}

return dataValid;

}

Listing 4. Odczytanie wyniku konwersji

double Measure_V (void)

{

uint16_t buf[2];

double voltage;

GetPDAC(buf);

voltage = (double)buf[1];

voltage = (voltage/83.6)  *  0.5;

return voltage;

}

Listing 5. Procedura pomiaru napięcia wyjściowego

double Measure_I (void)

{

double current;

GetPDAC(buf);

current = (current/203.5) * 50;

}

return current ;

}

Listing 6. Procedura pomiaru prądu wyjściowego

void DAC2_Initialize(void)

{

// DACREF DVREF+; DACFM Right; DACEN enabled; DACTSEL CMP1; DACTRIG disabled; DACSLP disabled; DACSIDL disabled;

DAC2CON = 0x8001;

}

Listing 7. Konfiguracja przetwornika DAC2

void DAC2_OutputSet (uint16_t  inputData)

{

DAC2DAT  =  inputData;

}

Listing 8. Wyzwolenie konwersji DAC2

void SetV (uint8_t stepv)

{

DAC1_OutputSet( VSet[stepv] + VSetCor [stepv] );

}

//tablica ustawiania napięcia zasilacza

const int16_t VSet [] =

{

0,21,42,63,84,

105,126,147,168,

189,210,231,252,

273,294,315,336,

357,378,399,420,

441,462,483,504,

525,546,567,588,

609,630,651,672,

693,714,735,756,

777,798,819,840,

861,882,903,924,

945,966,987,1008,

};

Listing 9. Procedura ustawiania napięcia wyjściowego z tablicą VSet

while (1)

{

//pomiar i wyświetlenie  prądu wyjściowego

current = Measure_I();

HMI_CurrentDisplayFast (16, 53,current, RED, WHITE);

//pomiar i wyświetlenie  napięcia wyjściowego

volt = Measure_V();

HMI_VoltageDisplayFast (16,10,volt, BLUE, WHITE);

//odczyt enkodera ustawiania napięcia

key_v = HMI_GetEncoderV();

//odczyt enkodera ustawiania ograniczenia prądowego

key_i = HMI_GetEncoderI();

//obsługa ustawienia napięcia

if(key_v == KOD_IMP_UP_V)

{

++Vstep;

if (Vstep > 48)

Vstep = 48;

SetV (Vstep);

volt = Measure_V();

HMI_VoltageDisplayFast (16, 10, volt , BLUE, WHITE);

}

if(key_v == KOD_IMP_DWN_V)

{

--Vstep;

if (Vstep < 1)

Vstep = 1;

SetV (Vstep);

volt = Measure_V();

HMI_VoltageDisplayFast (16, 10, volt, BLUE, WHITE)

}

//obsługa ustawienia prądu zabezpieczenia prądowego

if(key_i == KOD_IMP_UP_I)

{

++Istep;

if(Istep > 19)

Istep = 19;

SetI(Istep);

current_set = Istep * 50;

HMI_OverCurrentDisplay (19, 100, current_set, GOLD, BLACK);

}

if(key_i == KOD_IMP_DWN_I)

{

--Istep;

if(Istep <1 )

Istep = 1;

SetI(Istep);

current_set = Istep * 50;

HMI_OverCurrentDisplay (19, 100, current_set, GOLD, BLACK);

}

Sprawdzanie przyciśnięcia przycisku START i przełączanie przekaźnika wyjściowego

HMI_Check_START_Button();

}

Listing 10. Pętla główna programu.