# Plik http_server.py import _thread # 1 import esp32 import gc import neopixel import network import socket import sys import time import wifi_config from machine import Pin def led_task(gpio_num, delay_ms): # 2 led = Pin(gpio_num, Pin.OUT) # 3 while True: # 4 led(not led()) # 5 time.sleep_ms(delay_ms) # 6 def wifi_connect(): # 7 global station station = network.WLAN(network.STA_IF) station.active(True) if not station.isconnected(): print("Łączenie z siecią", end="") station.connect(wifi_config.ssid, wifi_config.password) while not station.isconnected(): print(".", end="") time.sleep_ms(250) print() global ip # 8 ip = station.ifconfig()[0] print(f"Adres IP: {ip}") def index_html(): # 9 gc.collect() # 10 content = "" with open("index.html", encoding="utf-8") as file: # 11 content += file.read() if led[0] == (0x10, 0x00, 0x00): # 12 color = "Czerwony" elif led[0] == (0x10, 0x10, 0x00): color = "Żółty" elif led[0] == (0x00, 0x10, 0x00): color = "Zielony" elif led[0] == (0x00, 0x10, 0x10): color = "Błękitny" elif led[0] == (0x00, 0x00, 0x10): color = "Niebieski" elif led[0] == (0x10, 0x00, 0x10): color = "Fioletowy" elif led[0] == (0x10, 0x10, 0x10): color = "Biały" else: color = "Czarny" content = content.replace("AA", str(esp32.mcu_temperature())) # 13 content = content.replace("BBB", str(station.status("rssi"))) content = content.replace("CCCCCCCCC", color) return content def http_task(): # 14 sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM) # 15 sock.bind(("", 80)) # 16 sock.listen() while True: try: # 17 gc.collect() # 18 conn, addr = sock.accept() # 19 request = conn.recv(1024) # 20 if request == b"": # 21 conn.send("HTTP/1.1 400 Bad Request\r\n") conn.send("Connection: close\r\n") conn.sendall("\r\n") conn.close() continue request = request.splitlines()[0] # 22 print(f"HTTP Request from {addr[0]}: {request}") if b"GET / HTTP" in request: # 23 conn.send(f"HTTP/1.1 307 Temporary Redirect\r\n") conn.send(f"Location: http://{ip}/index.html\r\n") conn.send("Connection: close\r\n") conn.sendall("\r\n") elif b"index.html " in request: # 24 conn.send("HTTP/1.1 200 OK\r\n") conn.send("Content-Type: text/html\r\n") conn.send("Connection: close\r\n") conn.send("\r\n") conn.sendall(index_html()) # 25 elif b"color" in request: # 26 if b"red" in request: led[0] = (0x10, 0x00, 0x00) elif b"yellow" in request: led[0] = (0x10, 0x10, 0x00) elif b"green" in request: led[0] = (0x00, 0x10, 0x00) elif b"cyan" in request: led[0] = (0x00, 0x10, 0x10) elif b"blue" in request: led[0] = (0x00, 0x00, 0x10) elif b"magenta" in request: led[0] = (0x10, 0x00, 0x10) elif b"white" in request: led[0] = (0x10, 0x10, 0x10) else: led[0] = (0x00, 0x00, 0x00) conn.send("HTTP/1.1 200 OK\r\n") conn.send("Content-Type: text/html\r\n") conn.send("Connection: close\r\n") conn.send("\r\n") response = index_html() conn.sendall(response) led.write() # 27 else: # 28 print("Unknown request") conn.send("HTTP/1.1 404 Not Found\r\n") conn.sendall("\r\n") conn.close() # 29 except Exception as e: # 30 sys.print_exception(e) led = neopixel.NeoPixel(Pin(38, Pin.OUT), 1) # 31 led[0] = (0, 0, 0) led.write() wifi_connect() # 32 _thread.start_new_thread(led_task, [21, 1000]) # 33 _thread.start_new_thread(led_task, [47, 500]) _thread.start_new_thread(led_task, [48, 250]) _thread.start_new_thread(led_task, [45, 100]) _thread.start_new_thread(http_task, ()) # 34

Listing 2. Kod pliku http_server.py